-----------------------------------------------------------------------

-----------------------------------------------------------------------

DOCUMENT:

SOUND MASTER II BOARD DETECTION PROGRAM

Version 1.1

February 28, 1991

Programming and Documentation by Ryan Hanlon

Copyright (c) 1991,  Covox, Inc.  All Rights Reserved

----------------------------------------------------------------------

PROGRAM NAME : sm2detec.exe

----------------------------------------------------------------------

PROGRAM DESCRIPTION

This program determines if a Sound Master II is  present in the computer

by doing an auto-detect for each of the three  devices on the SM II

(i.e., MIDI, VMDMA and YM3812).

The primary auto-detect functions within the program are :

1. MidiPortDetect    - Midi interface

2. YM3812PortDetect  - Yamaha 3812 control (Adlib compatible section).

3. VMDMAPortDetect   - Voice Master section.

SM2DETEC.C was designed to test the above listed auto-detect functions

and not as a stand alone application. Therefore main() can be removed

from the code without consequence.

All constants referred to in this document can be found in sm2detec.h

----------------------------------------------------------------------

COMPILATION REQUIREMENTS

This program was designed to be complied with Microsoft C version 6.0

or Turbo C version 2.0.

To compile with MICROSOFT C issue the following command at the DOS prompt.

C:\> cl /Ism2detec.h sm2detec.c;

To compile with TURBO C issue the following commands.

C:\> tcc -c -B sm2detec.c

.

.

.

C:> tlink -x -Ism2detec.h \tc\lib\c0s + sm2detec,sm2detec

,,\tc\lib\cs.lib

----------------------------------------------------------------------

AUTO-DETECT FUNCTION PROTOTYPES

1. MidiPortDetect

PROTOTYPE   : unsigned MidiPortDetect(char * Midi_Interrupt_Vector);

RETURNS  port address : Value returned is the port address of the MIDI

on the Sound Master II board.  Valid port

addresses are 0x330 and 0x338.

0  : Device not present or jumpers are not installed

properly.

PARAMETERS  : Midi_Interrupt_Vector : This parameter is the IRQ vector

address the MIDI is using.  It is

set if the function is successful

in detecting a port address.

DESCRIPTION : This function will return the port value that the Midi

section of the Sound Master II is currently using.  If the

Sound Master II is not found or there is a problem with the

board then a zero is returned. The problems with the board

that may exist are improper installation (e.g., jumper

setting conflicts), or hardware failure.

The parameter Midi_Interrupt_Vector is set and must be

referred to by address.

EXAMPLE     :

/* Call function to perform auto-detect on MIDI on the

// Sound Master II.

*/

Midi_Port_Value = MidiPortDetect( &Midi_Interrupt_Vector );

if( Midi_Port_Value )

{

/* Print message indicating detected Port and IRQ values.

*/

printf("MIDI using port       -->  %x\n", Midi_Port_Value );

printf("MIDI using IRQ vector -->  %x\n", Midi_Interrupt_Vector );

}

else

{

/* Print message indicating that the MIDI auto-detect failed.

*/

printf( "MIDI detection failed.\n" );

}

----------------------------------------------------------------------

2. YM3812PortDetect

PROTOTYPE   : unsigned YM3812PortDetect ( void );

RETURNS  port address : Value returned is the port address of the Yamaha

3812 Music Synthisizer (Adlib compatible section),

on the Sound Master II board.  Valid port

addresses are 0x380 and 0x388.

0 : Device not present or jumpers are not installed

properly.

PARAMETERS  : None.

DESCRIPTION : This function will return the port value that the YM3812

section of the Sound Master II is currently using.  If the

Sound Master II is not found or there is a problem with the

board then a zero is returned.  The problems with the board

that may exist are improper installation (e.g., jumper

setting conflicts), or hardware failure.

EXAMPLE     :

/* Call function to perform auto-detect on YM3812 on the

// Sound Master II.

*/

YM3812_Port_Value = YM3812PortDetect();

if( YM3812_Port_Value )

{

/* Print message indicating detected YM3812 Port on SM II.

*/

printf("YM3812 using port  -->  %x\n", YM3812_Port_Value );

}

else

{

/* Print message indicating that the YM3812 auto-detect

// failed.

*/

printf( "YM3812 detection failed.\n" );

}

----------------------------------------------------------------------

3. VMDMAPortDetect

PROTOTYPE   : VMDMAPortDetect(char * VMDMA_Interrupt_Vector,

char * VMDMA_Channel );

RETURNS  port address : Value returned is the port address of the VMDMA

on the Sound Master II board.  Valid port

addresses are 0x220, 0x240, 0x280 and 0x2C0.

0  : Device not present or jumpers are not installed

properly.

PARAMETERS  : VMDMA_Interrupt_Vector : This parameter is the IRQ vector

address the VMDMA is using and is

set if the function is successful

in detecting a port address.

VMDMA_Channel          : This parameter is the DMA channel

the VMDMA is using and is set if

the function is successful in

detecting a port address.

DESCRIPTION : This function will return the port value that the VMDMA

section of the Sound Master II is currently using.  If the

Sound Master II is not found or there is a problem with the

board then a zero is returned. The problems with the board

that may exist are improper installation (e.g., jumper

setting conflicts), or hardware failure.

The parameter VMDMA_Interrupt_Vector is set and must be

referred to by address.

The parameter VMDMA_Channel is set and must be referred to

by address.

EXAMPLE     :

/* ------ VOICE MASTER PORT DETECT

*/

VMDMA_Port_Value = VMDMAPortDetect(&VMDMA_Interrupt_Vector, &VMDMA_Channel);

if( VMDMA_Port_Value )

{

/* Print message indicating detected Port, Channel, and IRQ.

*/

printf("VMDMA using port    -->  %x\n", VMDMA_Port_Value );

printf("VMDMA using IRQ     -->  %x\n", (VMDMA_Interrupt_Vector-8) );

printf("VMDMA using Channel -->  %x\n", VMDMA_Channel );

}

else

{

/* Print message indicating that the VMDMA auto-detect

*/ failed to find a Voice Master or Sound Master II board.

*/

printf( "VMDMA detection failed.\n" );

}